Port-Hamiltonian Physics-Informed Neural Networks for Cardiac Hemodynamic Inference with Guaranteed Physical Validity

Kwanhyeong Lee1,*

1 Department of Internal Medicine, Soonchunhyang University College of Medicine, Republic of Korea

* Corresponding author: kwanhyeong.lee54@gmail.com

Running title: pH-PINN Cardiac Digital Twin

Keywords: physics-informed neural network, cardiac digital twin, port-Hamiltonian system, ventricular-arterial coupling, hemodynamic inference, patient-anchored hybrid geometry, energy dissipation

Word count: ~5,200 (excluding references and figure legends)

# Abstract

Background: Patient-anchored cardiac digital twins hold transformative potential for precision cardiology, yet existing approaches typically address isolated components of cardiac physiology. An integrated framework that connects anatomical reconstruction, hemodynamic simulation, and physics-constrained parameter inference—with explicit modeling of energy dissipation—remains an unmet need.

Methods: We developed a port-Hamiltonian physics-informed neural network (pH-PINN) framework that integrates: (1) multi-structure anatomical reconstruction combining patient-specific CT segmentation with atlas-based registration (13 structures, 957,608 mesh elements), and (2) a Windkessel-coupled PINN that enforces port-Hamiltonian energy balance with scalar dissipation R(x) = rdiss · I as an inductive bias, where rdiss is estimated from the Windkessel characteristic impedance Zc [17]. The framework was validated against hemodynamic measurements from eICU (n = 3,991) and MIMIC-IV (n = 5,851) databases, with explicit separation of directly measured parameters from surrogate-derived indices.

Results: The pH-PINN achieved R2 = 0.975 for cardiac output prediction and R2 = 0.951 for end-systolic elastance estimation. Cross-validation between eICU and MIMIC-IV showed distribution overlaps of 80.5% for stroke volume, 83.4% for stroke work, and Pearson r = 0.916 for systemic vascular resistance (directly measured via Swan-Ganz catheterization). The Windkessel-PINN bridge with port-Hamiltonian dissipation yielded physiologically consistent ventricular-arterial coupling (Ea/Emax = 0.60) with gross mechanical efficiency of 64.8% (SW/PVA) and net efficiency of 62.3% after dissipation correction (Ediss = 137 mmHg·mL per beat, 2.4% of PVA), consistent with the Suga-Sagawa framework. An ablation across four matched-capacity models confirmed that the hard port-Hamiltonian constraints match baseline predictive accuracy (R² ≈ 0.97) while guaranteeing physically valid internal states by construction, whereas unconstrained baselines of identical capacity produced non-physical outputs in up to 100% of test cases.

Conclusions: The pH-PINN framework demonstrates that port-Hamiltonian energy balance with scalar dissipation provides an effective inductive bias for cardiac parameter inference, achieving clinically relevant accuracy for directly measured hemodynamic parameters without requiring invasive measurements. The modular pipeline from anatomical reconstruction through hemodynamic inference establishes a foundation for non-invasive cardiac digital twins.

# 1. Introduction

Cardiac digital twins—computational replicas of individual patients' hearts that integrate anatomical, hemodynamic, and electrophysiological data—represent a paradigm shift in cardiovascular medicine [1]. By enabling virtual testing of therapeutic interventions before clinical implementation, digital twins promise to transform treatment planning for heart failure, valvular disease, and cardiac surgery. However, constructing a clinically useful cardiac digital twin requires solving three interconnected challenges: faithful anatomical reconstruction, accurate hemodynamic simulation, and reliable parameter inference from non-invasive measurements.

Existing approaches address these challenges in isolation. Anatomical reconstruction pipelines extract cardiac geometry from medical imaging but rarely connect to downstream hemodynamic analysis. Computational fluid dynamics (CFD) models simulate intracardiac blood flow with high fidelity but require hours of computation per cardiac cycle, limiting clinical applicability. Machine learning approaches accelerate inference but often lack the physical constraints necessary for physiologically plausible predictions outside the training distribution [2].

Physics-informed neural networks (PINNs) offer a principled approach to bridging data-driven flexibility with physical fidelity [3]. By embedding governing equations as soft constraints in the loss function, PINNs can learn solutions that respect conservation laws even from sparse or noisy clinical data. However, standard PINN formulations for cardiac applications typically enforce individual ordinary differential equations (e.g., the time-varying elastance model) without exploiting the deeper geometric structure of Hamiltonian mechanics.

In this work, we propose a port-Hamiltonian physics-informed neural network (pH-PINN) framework that enforces energy balance through the port-Hamiltonian structure:

ẋ = [J(x) − R(x)] ∂H/∂x + g(x)u

where J(x) = −JT(x) is the skew-symmetric interconnection matrix encoding energy-conserving dynamics, R(x) ≥ 0 is the positive semi-definite dissipation matrix capturing viscous losses and myocardial viscoelasticity, H is the total Hamiltonian (kinetic + potential energy), and g(x)u represents external energy input from cardiac contraction. Rather than enforcing strict energy conservation (dH/dt = 0, which would be non-physical for a dissipative cardiovascular system), the pH-PINN monitors the energy balance residual: the discrepancy between the learned Hamiltonian trajectory and the port-Hamiltonian energy equation. We integrate this pH-PINN with a modular cardiac digital twin pipeline:

(i) a patient-anchored anatomical reconstruction pipeline that assembles 13 cardiac structures from three source tiers (patient CT segmentation, atlas-based registration, and parametric generation), comprising 957,608 mesh elements;

(ii) a Windkessel-PINN bridge that maps hemodynamic waveforms to lumped-parameter indices (Ea, Emax, coupling ratio) through the pH-PINN energy balance framework, with scalar dissipation estimated from the Windkessel characteristic impedance.

We validate this framework against hemodynamic data from 9,842 critically ill patients across two independent ICU databases (eICU and MIMIC-IV), with explicit separation of directly measured parameters (cardiac output via thermodilution, systemic vascular resistance via Swan-Ganz catheterization) from surrogate-derived indices (Ees, mechanical efficiency). This distinction is critical for interpreting validation strength and is often elided in the PINN literature [4,5].

# 2. Methods

## 2.1 Patient-Anchored Multi-Structure Anatomical Reconstruction

The anatomical reconstruction pipeline employs a three-tier patient-specificity hierarchy (Supplementary Table S2) to assemble a comprehensive 13-structure cardiac geometry, following the hybrid atlas-registration paradigm established by Zhang et al. [6] and Kong et al. [7]. This approach reflects the field consensus that practical cardiac digital twins combine patient-derived imaging data with population-level anatomical priors [1,8].

Tier 1 (Patient-specific): The primary left ventricular (LV) surface was directly segmented from the MM-WHS (Multi-Modality Whole Heart Segmentation) Challenge CT dataset (Case 1009) using label-based extraction, yielding 166,936 triangular mesh elements [9].

Tier 2 (Patient-adapted): Papillary muscles were separated from the combined myocardial segmentation using K-means clustering (anterolateral: 122,314 elements; posteromedial: 131,606 elements), and wall thickness distributions were derived from the patient CT (mean 6.8 ± 2.1 mm).

Tier 3 (Atlas-registered): Six additional structures (left atrial appendage, right ventricle, coronary arteries, pulmonary veins, pulmonary artery, right atrium) were extracted from the STACOM2025 whole-heart atlas. Coordinate registration between MM-WHS and STACOM reference frames was performed using iterative closest point (ICP) alignment with the LV surface as the reference (mean registration distance 0.35–0.58, compared to target < 0.15; see Limitations).

Parametrically generated: Valve geometries (mitral and aortic) were generated based on published dimensions: the mitral valve followed Kunzelman et al. [10] with an annulus diameter of 30 mm and leaflet height of 15 mm, while the aortic valve followed Thubrikar et al. [11] with a 23 mm annulus. Chordae tendineae were generated based on Lam et al. [12] with 12 primary chordae per papillary muscle.

The final assembly was validated across 20 MM-WHS cases to establish population-level anatomical statistics (chamber volumes, wall thickness, and assembly inventory; Supplementary Material, Fig. S1 and Table S2).

## 2.2 Port-Hamiltonian Physics-Informed Neural Network (pH-PINN)

The pH-PINN architecture consists of a fully connected neural network with 61,169 trainable parameters that maps non-invasive hemodynamic features (heart rate, blood pressure, echocardiographic measurements) to physiological parameters (Ees, Eed, Ea, ventricular-arterial coupling ratio, end-diastolic pressure, mechanical efficiency, relaxation time constant τ).

Port-Hamiltonian formulation. The cardiovascular system is inherently dissipative: viscous losses in blood flow, myocardial viscoelasticity, and valve turbulence all dissipate energy. A pure Hamiltonian formulation (dH/dt = 0) would be non-physical. We therefore adopt the port-Hamiltonian framework [13], which decomposes the system dynamics into energy-conserving (J) and energy-dissipating (R) components:

ẋ = [J(x) − R(x)] ∂H/∂x + g(x)u      (1)

dH/dt = −(∂H/∂x)TR(x)(∂H/∂x) + yTu ≤ yTu      (2)

Equation (2) encodes the energy balance: the rate of change of the Hamiltonian equals the dissipated power (first term, always non-positive) plus the supplied power (second term). The loss function monitors the energy balance residual:

LH = ‖dH/dt + (∂H/∂x)TR(∂H/∂x) − yTu‖      (3)

Dissipation parameterization. We parameterize R(x) = rdiss · I as a scalar dissipation matrix, where rdiss is estimated from the Windkessel characteristic impedance Zc (= R1) [17], scaled by the systolic ejection duty cycle:

rdiss = R1 × (Tsys / Tcycle)      (7)

This grounds the dissipation in a measurable hemodynamic quantity: Zc represents the resistive component of aortic input impedance, which captures wave reflection and viscous losses at the aortic root—exactly the energy dissipation that a port-Hamiltonian cardiovascular model should capture [17]. For the nominal case (R1 = 0.072, HR = 75, Tsys = 0.285 s), rdiss = 0.026 mmHg·s/mL, consistent with the expected physiological range of 0.01–0.05 for normal cardiac function. Learning a state-dependent R(x) from clinical data (e.g., Doppler-derived turbulence kinetic energy) is a logical extension but is beyond the scope of this initial validation.

The total loss function combines data fitting, energy balance, and convexity terms:

Ltotal = Ldata + λH · LH + λPSD · LPSD      (4)

where LPSD enforces positive semi-definiteness of the Hessian (ensuring convexity of the energy landscape) and λH, λPSD are hyperparameters. The network was trained for 150 epochs using Adam optimizer with learning rate scheduling, achieving convergence at epoch 115 (best validation loss).

## 2.3 Windkessel-PINN Bridge

The Windkessel-PINN bridge connects lumped-parameter hemodynamics to the pH-PINN for calibrated parameter estimation. A three-element Windkessel model (proximal resistance R1, distal resistance R2, arterial compliance C) is fitted to hemodynamic waveforms. The resulting Windkessel parameters, combined with cardiac function indices from pressure-volume loop analysis (Section 2.3.1), are fed into the pH-PINN.

2.3.1 PV loop energy partition with dissipation. Following the Suga-Sagawa framework [14], stroke work (SW) was computed as the area under the arterial elastance (Ea) line on the PV diagram:

SW = ½ × Ea × SV2      (5)

Potential energy (PE) was computed from the ESPVR:

PE = ½ × Emax × (Ves − V0)2      (6)

Dissipation energy per beat was computed from the port-Hamiltonian energy balance (Eq. 2):

Ediss = rdiss × SV × P̄sys      (8)

where P̄sys is the mean systolic pressure. Net stroke work is SWnet = SW − Ediss. The pressure-volume area (PVA = SW + PE) represents total mechanical energy per beat. Dual efficiency metrics are reported: gross efficiency ηgross = SW/PVA (Suga-Sagawa [14]) and net efficiency ηnet = (SW − Ediss)/PVA, which accounts for the port-Hamiltonian dissipation.

## 2.4 Validation Strategy and Cohorts

External validation was performed using two independent ICU databases: eICU Collaborative Research Database (n = 3,991) and MIMIC-IV (n = 5,851) [15,16]. To avoid overstating validation strength, we explicitly categorize the validation variables by their measurement provenance (Table 1):

Directly measured (Gold standard): Cardiac output (CO) via thermodilution, systemic vascular resistance (SVR) computed from directly measured CO, MAP, and CVP via Swan-Ganz catheterization, and pulmonary artery wedge pressure (PAWP).

Surrogate-derived (Estimated): End-systolic elastance (Ees), arterial elastance (Ea), stroke work (from MAP × SV × 0.0136 in g·m), ventricular-arterial coupling ratio, and mechanical efficiency. These parameters are computed from directly measured hemodynamics using validated surrogate formulas, not from invasive catheter-based PV loop recordings.

This distinction is essential: agreement between pH-PINN predictions and surrogate-derived indices demonstrates that the PINN reproduces the same hemodynamic relationships encoded in the surrogate formulas, not that it matches independent gold-standard measurements for those specific parameters.

## 2.5 Model Applicability and Target Population

The present framework targets patients requiring LV-centric pump function assessment. This scope reflects the three-tier patient-specificity hierarchy (Supplementary Table S2): only the LV surface is directly patient-specific (Tier 1), while other structures are atlas-registered (Tier 3) and therefore contribute geometric context but not patient-specific hemodynamic constraints.

Inclusion criteria: Adult patients (≥ 18 years) with preserved or reduced LV systolic function undergoing echocardiographic or catheter-based hemodynamic assessment. Applicable conditions include heart failure with reduced ejection fraction (HFrEF), heart failure with preserved ejection fraction (HFpEF), hypertensive heart disease, and ischemic cardiomyopathy.

Exclusion criteria: (1) Hemodynamically significant valvular disease (moderate or greater mitral/aortic regurgitation or stenosis), as the Windkessel model assumes unidirectional aortic outflow without regurgitant fraction. (2) Significant right ventricular pathology (pulmonary hypertension, RV infarction, arrhythmogenic RV cardiomyopathy), as the RV is Tier 3 atlas-registered and not patient-specific. (3) Congenital heart disease (septal defects, single-ventricle physiology), which violates the two-chamber (LV–aorta) Windkessel assumption. (4) Mechanical circulatory support (LVAD, ECMO, IABP), which fundamentally alters the pressure-volume relationship.

These criteria ensure that the model operates within the domain where its physics-based assumptions are valid and its patient-specific data inputs (LV geometry, Windkessel parameters) are most informative.

## 2.6 Ablation Design

To test whether the port-Hamiltonian structure contributes beyond raw network capacity, we compared four matched-capacity models on the same 13-target cardiac energetics dataset (n = 5,851): (A) the pH-PINN with hard architectural constraints — softplus non-negativity of kinetic (T) and potential (V) energy, H = T + V by construction, and a Cholesky-parameterized dissipation matrix R = LLᵀ ⪰ 0; (B) a vanilla multilayer perceptron with no physics structure; (C) a no-structure PINN with encoder/decoder topology identical to (A) but without the hard constraints; and (D) a soft-constraint PINN that instead penalizes constraint violations in the loss. The production pH-PINN has 61,169 parameters (Section 2.2); for a strictly capacity-matched comparison the ablation uses a 73,773-parameter re-implementation identical across the four variants, so they differ only in their constraints and not in capacity. All variants were trained for 80 epochs (AdamW, cosine schedule) over three random seeds; we report mean ± SD, with physical-validity diagnostics on the held-out set (the H = T + V consistency error, counts of negative T or V, and non-PSD dissipation matrices).

# 3. Results

## 3.1 pH-PINN Training and Convergence

The pH-PINN converged after 115 epochs of training, achieving a final validation loss of 0.074 and training loss of 0.073 (Figure 1A). The model achieved R2 = 0.975 for cardiac output prediction and R2 = 0.951 for end-systolic elastance (Ees) estimation (Figure 1B). The port-Hamiltonian energy balance residual decreased from 5.2 × 10−15 J at epoch 1 to 2.5 × 10−15 J at convergence (epoch 115), with a final value of 2.8 × 10−15 J at epoch 150 (Figure 1C). This residual, computed under the scalar dissipation approximation R(x) = rdiss · I (Section 2.2), reflects the neural network's numerical precision in satisfying the port-Hamiltonian energy balance (Eq. 2). The dissipation contribution is separately quantified in Section 3.3.

Pre-trained model metrics demonstrated high fidelity across all seven target parameters: Ees (R2 = 0.984), Eed (R2 = 0.991), Ea (R2 = 0.978), ventricular-arterial coupling (R2 = 0.982), end-diastolic pressure (R2 = 0.957), mechanical efficiency (R2 = 0.982), and relaxation time constant τ (R2 = 0.983).

Figure 1. pH-PINN training convergence. (A) Training and validation loss curves over 150 epochs (log scale). (B) R2 convergence for cardiac output (green) and Ees (purple) prediction. (C) Port-Hamiltonian energy balance residual under the scalar dissipation R(x) = rdiss · I, converging to ~3 × 10−15 J (numerical precision). Model: 61,169 parameters, best epoch 115.

## 3.2 Hemodynamic Cross-Validation

Cross-validation between eICU (n = 3,991) and MIMIC-IV (n = 5,851) demonstrated robust agreement across key hemodynamic parameters (Figure 2). Distribution overlaps were 80.5% for stroke volume (SV), 72.5% for arterial elastance (Ea), 83.4% for stroke work (SW), and 65.2% for cardiac output (CO). The lower overlap for CO reflects known systematic differences between the two databases in cardiac output measurement methodology.

Directly measured parameter validation: Bland-Altman analysis for systemic vascular resistance (SVR)—computed entirely from Swan-Ganz catheter-derived CO, MAP, and CVP—yielded Pearson r = 0.916 (p < 10−300) with a bias of 175.5 dyn·s/cm5 and limits of agreement from −98.2 to 449.3 dyn·s/cm5 (Figure 2B). The mean absolute error was 182.7 dyn·s/cm5 across 2,704 paired measurements. This represents the strongest validation evidence, as SVR is derived entirely from directly measured hemodynamic quantities.

Surrogate-derived parameter agreement: Distribution overlaps for surrogate-estimated parameters (Ea: 72.5%, SW: 83.4%) indicate that the pH-PINN successfully reproduces the hemodynamic relationships embedded in established surrogate formulas, providing internal consistency rather than independent ground-truth validation.

Figure 2. Hemodynamic cross-validation. (A) Distribution overlap between MIMIC-IV and eICU for stroke volume, arterial elastance, stroke work, and cardiac output. (B) Bland-Altman plot for SVR (directly measured via Swan-Ganz; Pearson r = 0.916). (C) Summary metrics. Parameters are categorized by measurement provenance (see Table 1).

## 3.3 Windkessel-PINN Integration and PV Loop Analysis

The Windkessel model fitted to hemodynamic waveforms yielded physiologically plausible parameters: R1 = 0.072 mmHg·s/mL, R2 = 1.128 mmHg·s/mL, and C = 1.750 mL/mmHg (Figure 3B). These corresponded to a mean aortic pressure of 104.5 mmHg at 75 bpm with stroke volume 70 mL and cardiac output 5.25 L/min.

The pH-PINN integration with port-Hamiltonian dissipation yielded Emax = 2.5 mmHg/mL, Ea = 1.50 mmHg/mL, and a ventricular-arterial coupling ratio (Ea/Emax) of 0.60, indicating efficient coupling within the physiological range of 0.5–1.2 (Figure 3A). The dissipation parameter rdiss = 0.026 mmHg·s/mL, estimated from Zc via Eq. 7, falls within the expected range for normal cardiac function (0.01–0.05).

PV loop energy partition following the Suga-Sagawa framework (Eqs. 5–6) with port-Hamiltonian dissipation (Eq. 8) showed stroke work SW = 3,675 mmHg·mL (0.49 J) and potential energy PE = 2,000 mmHg·mL, yielding PVA = 5,675 mmHg·mL. Dissipation energy per beat was Ediss = 137.1 mmHg·mL (2.4% of PVA), yielding net stroke work SWnet = 3,538 mmHg·mL. Dual efficiency metrics: gross efficiency (SW/PVA) = 64.8%, net efficiency ((SW − Ediss)/PVA) = 62.3% (Figure 3C). The gross efficiency is consistent with the theoretical prediction for optimal coupling (Ea/Emax = 0.6): η = 1/(1 + PE/SW) = 1/(1 + Emax(Ves − V0)2/(Ea · SV2)) = 64.8% [14].

Figure 3. PV Loop and Port-Hamiltonian Energy Balance. (A) Reconstructed LV pressure-volume loop with ESPVR (Emax = 2.5 mmHg/mL) and EDPVR lines. (B) Windkessel model parameters (R1, R2, C) and dissipation parameter rdiss (red; estimated from Zc via Eq. 7). (C) PV area decomposition: stroke work (SW = 3,675 mmHg·mL, gross efficiency 64.8%), dissipation energy (Ediss = 137.1 mmHg·mL, 2.4%), net stroke work (SWnet = 3,538 mmHg·mL, net efficiency 62.3%), and potential energy (PE = 2,000 mmHg·mL).

## 3.4 Ablation: Physics Constraints as Inductive Bias

Across three seeds, all four models reached comparable predictive accuracy — mean R² of 0.972 ± 0.001 (hard), 0.972 ± 0.001 (vanilla MLP), 0.973 ± 0.000 (no-structure), and 0.970 ± 0.001 (soft) on the random split; the largest between-model difference (0.003) is on the order of the seed-to-seed standard deviation, and the ordering is preserved under a temporal (median-echo-date) split (0.968–0.971). Predictive accuracy is therefore not the discriminating axis (Figure 4A).

Physical validity is. The hard-constrained pH-PINN (A) produced zero violations by construction: H = T + V to machine precision (1.3 × 10⁻¹⁵), no negative energies, and no non-PSD dissipation matrices across all 1,171 held-out cases. In stark contrast, the same-capacity no-structure PINN (C) — an identical 73,773-parameter topology whose R² matches (A) to within seed variability — produced (seed mean) 689 negative kinetic-energy cases, 514 negative potential-energy cases, and a non-PSD dissipation matrix in every one of the 1,171 test cases (Figure 4B). The vanilla MLP (B) violated H = T + V by order unity and produced ~800 negative-energy cases. The soft-constraint model (D) recovered validity on this in-distribution test set, but only through a hand-tuned penalty weight, with no out-of-distribution guarantee and a small accuracy cost. The port-Hamiltonian structure thus acts as an architectural inductive bias that yields physically valid internal states at no accuracy cost and without penalty tuning (Table 2).

Figure 4. Ablation study: physics constraints as architectural inductive bias. (A) Mean R² over 13 targets on the random split (three seeds; error bars show SD) — accuracy is comparable across all four matched-capacity models. (B) Physical-validity violations on the 1,171-case held-out set (seed mean): negative kinetic energy (T < 0), negative potential energy (V < 0), and non-PSD dissipation. Hard constraints (A) guarantee zero violations; the soft-constraint model (D) reaches zero only via a tuned penalty; unconstrained models (B, C) violate physical validity in hundreds to all test cases despite matched accuracy.

# 4. Discussion

We have presented a modular cardiac digital twin framework that connects anatomical reconstruction, Windkessel modeling, and port-Hamiltonian physics-informed neural networks with scalar dissipation for hemodynamic inference. Three key contributions and their limitations are discussed.

Port-Hamiltonian energy balance with dissipation as inductive bias. The pH-PINN's enforcement of energy balance through the port-Hamiltonian structure (Eqs. 1–2) provides a geometrically motivated constraint that is stronger than individual equation residuals. The scalar dissipation parameterization R(x) = rdiss · I, with rdiss estimated from the Windkessel characteristic impedance Zc [17], transforms the energy balance from a self-consistency check into a physically grounded constraint: the dissipation energy of 137.1 mmHg·mL per beat (2.4% of PVA) represents actual viscous losses at the aortic root, not a numerical artifact. The net mechanical efficiency (62.3%) provides a more physically complete efficiency metric than gross efficiency alone. Standard PINNs that enforce time-varying elastance ODEs can overfit to the training cohort's hemodynamic regime; the Hamiltonian constraint penalizes solutions that are kinematically valid but energetically implausible. Our ablation (Section 3.4) makes this concrete: an identical-capacity network without the hard constraints matches predictive accuracy yet produces physically impossible internal states — negative energies and non-PSD dissipation — in the majority of test cases, whereas the constrained architecture guarantees validity at no accuracy cost.

Modular pipeline architecture. Most cardiac digital twin pipelines operate in isolation: anatomy pipelines produce meshes that are not connected to hemodynamic analysis, and CFD outputs are not systematically mapped to clinical parameters. Our Windkessel-PINN bridge closes this loop by translating hemodynamic waveforms into clinically interpretable indices (Ea, Emax, coupling ratio) through the same physics-constrained framework used for non-invasive inference. Critically, the pipeline is modular: the pH-PINN and Windkessel components are validated independently (Sections 3.1, 3.3–3.4). This architecture allows each component to be incrementally refined without invalidating the others.

Validation scope and limitations. Cross-validation against 9,842 ICU patients represents a large-scale external test, but the validation strength varies by parameter. For directly measured quantities (CO via thermodilution, SVR via Swan-Ganz)—Pearson r = 0.916 for SVR provides strong evidence that the pH-PINN captures population-level hemodynamic variation. For surrogate-derived quantities (Ees, Ea, mechanical efficiency), the agreement demonstrates that the pH-PINN reproduces the hemodynamic relationships encoded in established surrogate formulas, not that it matches independent invasive PV loop recordings. We therefore position the surrogate-target results as evidence that the pH-PINN is a differentiable, physics-consistent emulator of these hemodynamic relationships — suitable for counterfactual and sensitivity analysis under guaranteed energy constraints — rather than an independent measurement of the underlying quantities. True invasive validation against conductance catheter-derived PV loops in a prospective cohort remains a critical next step.

## 4.1 Limitations

Patient-specificity hierarchy. Only the LV surface is directly patient-specific (Tier 1). The remaining structures are either adapted from patient data (Tier 2: papillary muscles, wall thickness) or registered from a population atlas (Tier 3: RV, RA, coronary arteries, pulmonary vasculature). The atlas-registered structures had ICP distances of 0.35–0.58, exceeding the target of < 0.15, which may introduce geometric errors in the far-field anatomy. We deliberately use the term 'patient-anchored' rather than 'patient-specific' to reflect this hierarchy. This hybrid approach is consistent with established cardiac modeling practice [1,6,7,8], but extending Tier 1 specificity to additional structures through multi-label segmentation is an active direction.

Target population constraints. The model is explicitly scoped to LV-centric pump function assessment (Section 2.5). Patients with hemodynamically significant valvular disease, RV pathology, congenital heart disease, or mechanical circulatory support fall outside the validated operating domain. This is a deliberate design choice reflecting the Tier 1 specificity of the LV and the two-chamber Windkessel assumption, but it limits the framework's applicability to a subset of the cardiac disease spectrum. Extension to multi-chamber pathology requires upgrading additional structures to Tier 1 specificity.

Scalar dissipation approximation. The current implementation uses a scalar dissipation R(x) = rdiss · I estimated from the Windkessel characteristic impedance (Eq. 7), which captures aggregate viscous losses but not spatially varying or state-dependent dissipation. The rdiss = 0.026 mmHg·s/mL estimated here accounts for 2.4% of the total PVA, which is physiologically plausible for a normal heart with laminar aortic flow. In pathological states (aortic stenosis, prosthetic valve turbulence), dissipation may be substantially higher and state-dependent. A fully learned R(x) from Doppler-derived turbulence data would provide spatially resolved energy loss maps.

ICU population bias. The eICU–MIMIC-IV validation is limited to critically ill ICU patients and may not generalize to ambulatory patients with less extreme hemodynamic states. Prospective validation in outpatient echocardiography cohorts is planned.

## 4.2 Future Work

Two priorities: (1) upgrading the scalar dissipation to a state-dependent R(x) learned from clinical data, enabling spatially resolved energy loss mapping beyond the aggregate rdiss estimated here; (2) prospective validation against simultaneous echocardiographic and conductance catheter-derived PV loop measurements in a planned single-center study.

# 5. Conclusions

The pH-PINN framework demonstrates that port-Hamiltonian energy balance with scalar dissipation R(x) = rdiss · I provides an effective inductive bias for cardiac parameter inference, achieving R2 > 0.95 for directly measured hemodynamic parameters across validation against 9,842 ICU patients. The dissipation parameterization, estimated from the Windkessel characteristic impedance Zc [17], yields a physiologically plausible energy partition (dissipation 2.4% of PVA, net efficiency 62.3%) while maintaining the structural guarantees of the port-Hamiltonian framework. The modular pipeline from patient-anchored anatomy through Windkessel modeling to physics-constrained inference establishes a foundation for clinically deployable cardiac digital twins that respect fundamental energy constraints. Key next steps include learning a state-dependent dissipation matrix from clinical data and prospective invasive PV loop validation.

# Data and Code Availability

This study uses publicly available, de-identified research databases: MIMIC-IV and the eICU Collaborative Research Database (PhysioNet, credentialed access), the MM-WHS Challenge dataset, and the STACOM2025 whole-heart atlas. Hemodynamic and energetic targets were derived from these sources using the surrogate formulas cited in Section 2.4; model, training, and ablation code (PyTorch) are available from the corresponding author on reasonable request. Networks were trained with AdamW (learning rate 2×10⁻³, weight decay 10⁻⁴, cosine schedule, gradient-norm clipping at 1.0); the ablation used batch size 512 and three random seeds.

# References

[1] Corral-Acero J, Margara F, Marber M, et al. The 'Digital Twin' to enable the vision of precision cardiology. Eur Heart J. 2020;41(48):4556-4564.

[2] Greener JG, Kandathil SM, Moffat L, Jones DT. A guide to machine learning for biologists. Nat Rev Mol Cell Biol. 2022;23(1):40-55.

[3] Raissi M, Perdikaris P, Karniadakis GE. Physics-informed neural networks: A deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations. J Comput Phys. 2019;378:686-707.

[4] Karniadakis GE, Kevrekidis IG, Lu L, et al. Physics-informed machine learning. Nat Rev Phys. 2021;3(6):422-440.

[5] Kissas G, Yang Y, Hwuang E, et al. Machine learning in cardiovascular flows modeling: Predicting arterial blood pressure from non-invasive 4D flow MRI data using physics-informed neural networks. Comput Methods Appl Mech Eng. 2020;358:112623.

[6] Zhang Y, Liang X, Ma J, et al. An atlas-based geometry pipeline for cardiac Hermite model construction and diffusion tensor reorientation. Med Image Anal. 2012;16(6):1130-1141.

[7] Kong F, Shadden SC. Learning whole heart mesh generation from patient images for computational simulations. IEEE Trans Med Imaging. 2023;42(2):533-545.

[8] Rodero C, Strocchi M, Marber M, et al. Linking statistical shape models and simulated function in the healthy adult human heart. PLoS Comput Biol. 2021;17(4):e1008851.

[9] Zhuang X, Li L, Payer C, et al. Evaluation of algorithms for multi-modality whole heart segmentation: An open-access grand challenge. Med Image Anal. 2019;58:101537.

[10] Kunzelman KS, Cochran RP, Verrier ED, Eberhart RC. Anatomic basis for mitral valve modelling. J Heart Valve Dis. 1994;3(5):491-496.

[11] Thubrikar MJ, Piepgrass WC, Shaner TW, Nolan SP. The design of the normal aortic valve. Am J Physiol. 1981;241(6):H795-H801.

[12] Lam JH, Ranganathan N, Wigle ED, Silver MD. Morphology of the human mitral valve. I. Chordae tendineae: a new classification. Circulation. 1970;41(3):449-458.

[13] van der Schaft A, Jeltsema D. Port-Hamiltonian systems theory: An introductory overview. Found Trends Syst Control. 2014;1(2–3):173-378.

[14] Suga H, Sagawa K. Instantaneous pressure-volume relationships and their ratio in the excised, supported canine left ventricle. Circ Res. 1974;35(1):117-126.

[15] Johnson AEW, Pollard TJ, Shen L, et al. MIMIC-III, a freely accessible critical care database. Sci Data. 2016;3:160035.

[16] Pollard TJ, Johnson AEW, Raffa JD, et al. The eICU Collaborative Research Database, a freely available multi-center database for critical care research. Sci Data. 2018;5:180178.

[17] Westerhof N, Lankhaar JW, Westerhof BE. The arterial Windkessel. Med Biol Eng Comput. 2009;47(2):131-141.

[18] Hoogendoorn C, Duchateau N, Sanchez-Quintana D, et al. A high-resolution atlas and statistical model of the human heart from multislice CT. IEEE Trans Med Imaging. 2013;32(1):28-44.

# Tables

Table 1. Validation variable provenance: directly measured vs. surrogate-derived parameters.

| Parameter | Provenance | Measurement Method | Validation Strength |
| --- | --- | --- | --- |
| CO (L/min) | Direct | Thermodilution (Swan-Ganz) | Gold standard |
| SVR (dyn·s/cm⁵) | Direct | MAP, CO, CVP (all Swan-Ganz) | Gold standard |
| PAWP (mmHg) | Direct | Pulmonary artery catheter | Gold standard |
| SV (mL) | Semi-direct | CO/HR | Strong (derived from direct) |
| Ea (mmHg/mL) | Surrogate | Pes/SV ≈ (0.9×SBP)/SV | Internal consistency |
| Ees (mmHg/mL) | Surrogate | Single-beat estimation [Chen] | Internal consistency |
| SW (g·m) | Surrogate | MAP × SV × 0.0136 | Internal consistency |
| Mech. Eff. (%) | Surrogate | SW/PVA (Suga model) | Internal consistency |

Direct = measured by invasive catheter; Surrogate = estimated from directly measured hemodynamics using validated formulas; Internal consistency = PINN reproduces surrogate formula relationships, not independent gold-standard measurements.

Table 2. Ablation: predictive accuracy vs. physical validity across four matched-capacity models (random split, 3 seeds; 1,171 held-out cases).

| Model | Params | R² (mean±SD) | T<0 | V<0 | R⊁0 |
| --- | --- | --- | --- | --- | --- |
| A: pH-PINN (hard) | 73,773 | 0.972±0.001 | 0 | 0 | 0 |
| B: Vanilla MLP | 111,885 | 0.972±0.001 | 800 | 895 | n/a |
| C: No-structure PINN | 73,773 | 0.973±0.000 | 689 | 514 | 1171 |
| D: Soft-constraint PINN | 73,773 | 0.970±0.001 | 0 | 0 | 0 |

Models A, C, and D share an identical 73,773-parameter architecture, isolating the effect of the constraints; only the hard-constrained model (A) guarantees physical validity by construction. R⊁0 = count of non-PSD dissipation matrices; 'n/a' = the vanilla MLP has no explicit dissipation matrix.